Fortinet Capstone Project

Enterprise Network Security Implementation

Documentation

Table of Contents

1. Project Overview	1

1.1 Objectives	1

1.2 Requirements Coverage	1

2. Network Architecture	1

2.1 Devices and Roles	1

2.2 Design Principles	1

3. Physical Connections	1

3.1 Internet and ISP Links	1

3.2 HQ	1

3.3 Branch 1	1

3.4 Branch 2	1

3.5 Management Network	1

4. IP Addressing Plan	1

4.1 WAN and Transit Links	1

4.2 LAN Subnets	1

4.3 End Devices	1

4.4 Management Addresses (port1)	1

5. VLAN Design	1

6. ISP and Edge Router Configuration	1

7. Switch Configuration	1

8. Headquarters Configuration	1

8.1 High Availability (Active-Passive)	1

8.2 Failover Test	1

8.3 Interfaces	1

8.4 DHCP for the User VLAN	1

8.5 Inter-VLAN Routing	1

9. Branch Configuration	1

9.1 Branch 1 (Fortinet4)	1

9.2 Branch 2 (Fortinet3)	1

10. Routing	1

11. IPsec Site-to-Site VPN	1

11.1 Parameters	1

11.2 Tunnels	1

11.3 Branch-to-Branch Communication	1

12. Firewall Policies	1

12.1 HQ	1

12.2 Branch 1	1

12.3 Branch 2	1

13. NAT Configuration	1

13.1 Outbound NAT	1

13.2 Virtual IP (Port Forwarding)	1

14. Security Profiles	1

15. Remote User VPN	1

16. Test Results	1

17. Challenges and Solutions	1

18. Conclusion	1

# 1. Project Overview

This project designs, configures and demonstrates a secure enterprise network built around FortiGate firewalls in an EVE-NG lab. The network simulates a real organization made of a Headquarters (HQ), two branch offices (Branch 1 and Branch 2) and a remote user, all connected through a simulated Internet made of ISP routers.

## 1.1 Objectives

- Secure segmentation with VLANs and inter-VLAN routing on the FortiGate.

- High availability at HQ with an Active-Passive FortiGate cluster.

- IPsec site-to-site VPN between HQ and both branches, including Branch-to-Branch communication through HQ.

- Firewall policies that follow the least-privilege principle.

- Outbound NAT for Internet access and a Virtual IP (VIP) that publishes an internal server.

- Security profiles: Antivirus, Web Filtering, IPS and Application Control.

- Remote access VPN for a remote user with authentication.

## 1.2 Requirements Coverage

| Requirement from the project brief | Where it is covered |
| --- | --- |
| HQ, Branch 1, Branch 2, Remote User, ISP simulation | Sections 2, 3, 4, 6 |
| HQ VLANs (User, Server, Management), trunk, inter-VLAN routing | Sections 5, 7, 8 |
| DHCP on the User VLAN | Section 8.4 |
| High Availability (Active-Passive) with failover test | Section 8.1, 8.2 |
| Branch VLANs and local inter-VLAN routing | Section 9 |
| IPsec HQ-Branch1, HQ-Branch2, Branch-to-Branch | Section 11 |
| Firewall policies, least privilege | Section 12 |
| NAT and VIP | Section 13 |
| Static routing | Section 10 |
| Security profiles | Section 14 |
| Remote user VPN | Section 15 |
| Test results | Section 16 |

# 2. Network Architecture

## 2.1 Devices and Roles

| Site | Device | Role |
| --- | --- | --- |
| HQ | vIOS6 | ISP router |
| HQ | vIOS5 | Edge/WAN router |
| HQ | Switch11 | WAN switch connecting the edge router to both FortiGates |
| HQ | Fortinet1 and Fortinet2 | FortiGate HA pair (Active-Passive) |
| HQ | Switch12 | Core switch (VLAN trunking) |
| HQ | Switch14 | Access switch for users (VPC17, VPC18) |
| HQ | Switch13 | Access switch for servers and management (VPC19) |
| Branch 1 | vIOS7 | ISP router and simulated Internet core (holds 8.8.8.8) |
| Branch 1 | vIOS8 | Edge/WAN router |
| Branch 1 | Fortinet4 (FGT-Branch1) | Branch firewall |
| Branch 1 | Switch15 | Access switch (VPC20, VPC21) |
| Branch 2 | vIOS9 | ISP router |
| Branch 2 | vIOS10 | Edge/WAN router |
| Branch 2 | Fortinet3 (FGT-Branch2) | Branch firewall |
| Branch 2 | Switch16 | Access switch (VPC22, VPC23) |
| Remote | Win | Windows client used for remote access VPN |

## 2.2 Design Principles

- port1 on every FortiGate is dedicated to out-of-band management (GUI and SSH) and never carries user or WAN traffic.

- port2 is the WAN interface with a static IP address; no address is obtained from the edge router by DHCP.

- port4 (HQ) and port3 (branches) carry tagged VLANs towards the LAN switches.

- port3 and port5 on the HQ FortiGates are the two heartbeat links of the HA cluster.

- Inter-VLAN routing is performed by the FortiGate at every site, so every inter-VLAN flow is inspected by a policy.

# 3. Physical Connections

## 3.1 Internet and ISP Links

| From | To |
| --- | --- |
| vIOS6 Gi0/1 | vIOS7 Gi0/1 |
| vIOS7 Gi0/2 | vIOS9 Gi0/1 |
| vIOS9 Gi0/2 | Win |

## 3.2 HQ

| From | To |
| --- | --- |
| vIOS6 Gi0/0 | vIOS5 Gi0/0 |
| vIOS5 Gi0/1 | Switch11 Gi0/0 |
| Switch11 Gi0/1 | Fortinet1 port2 |
| Switch11 Gi0/2 | Fortinet2 port2 |
| Fortinet1 port3 | Fortinet2 port3 (heartbeat 1) |
| Fortinet1 port5 | Fortinet2 port5 (heartbeat 2) |
| Fortinet1 port4 | Switch12 Gi0/0 |
| Fortinet2 port4 | Switch12 Gi0/1 |
| Switch12 Gi0/2 | Switch14 Gi0/0 |
| Switch12 Gi0/3 | Switch13 Gi0/0 |
| Switch14 Gi0/1 and Gi0/2 | VPC17 and VPC18 |
| Switch13 Gi0/1 | VPC19 |

## 3.3 Branch 1

| From | To |
| --- | --- |
| vIOS7 Gi0/0 | vIOS8 Gi0/0 |
| vIOS8 Gi0/1 | Fortinet4 port2 |
| Fortinet4 port3 | Switch15 Gi0/0 |
| Switch15 Gi0/1 and Gi0/2 | VPC20 and VPC21 |

## 3.4 Branch 2

| From | To |
| --- | --- |
| vIOS9 Gi0/0 | vIOS10 Gi0/0 |
| vIOS10 Gi0/1 | Fortinet3 port2 |
| Fortinet3 port3 | Switch16 Gi0/0 |
| Switch16 Gi0/1 and Gi0/2 | VPC22 and VPC23 |

## 3.5 Management Network

port1 of all four FortiGates connects to a management switch that is attached to a Cloud/NAT node, which gives access to every FortiGate GUI from the host machine (network 192.168.77.0/24).

# 4. IP Addressing Plan

The addressing scheme is logical and hierarchical: 203.0.x.x for provider-facing WAN links, 1.1.x.x for the simulated Internet core, and 10.<site>.<vlan>.0/24 for LAN subnets (site 10 = HQ, 20 = Branch 1, 30 = Branch 2).

## 4.1 WAN and Transit Links

| Link | Subnet | Addresses |
| --- | --- | --- |
| vIOS6 - vIOS5 | 203.0.1.0/30 | vIOS6 .1 / vIOS5 .2 |
| vIOS5 - HQ FortiGate cluster | 203.0.10.0/29 | vIOS5 .1 / HQ FortiGate .2 / VIP .3 |
| vIOS7 - vIOS8 | 203.0.2.0/30 | vIOS7 .1 / vIOS8 .2 |
| vIOS8 - Fortinet4 | 203.0.20.0/30 | vIOS8 .1 / Fortinet4 .2 |
| vIOS9 - vIOS10 | 203.0.3.0/30 | vIOS9 .1 / vIOS10 .2 |
| vIOS10 - Fortinet3 | 203.0.30.0/30 | vIOS10 .1 / Fortinet3 .2 |
| vIOS6 - vIOS7 | 1.1.1.0/30 | .1 / .2 |
| vIOS7 - vIOS9 | 1.1.2.0/30 | .1 / .2 |
| vIOS9 - Win | 5.5.5.0/24 | vIOS9 .1 / Win .2 |
| Simulated Internet host | 8.8.8.8/32 | Loopback0 on vIOS7 |

## 4.2 LAN Subnets

| Site | VLAN | Purpose | Subnet | Gateway |
| --- | --- | --- | --- | --- |
| HQ | 10 | Users | 10.10.10.0/24 | 10.10.10.1 |
| HQ | 20 | Servers | 10.10.20.0/24 | 10.10.20.1 |
| HQ | 30 | Management | 10.10.30.0/24 | 10.10.30.1 |
| Branch 1 | 10 | Users | 10.20.10.0/24 | 10.20.10.1 |
| Branch 1 | 20 | Servers | 10.20.20.0/24 | 10.20.20.1 |
| Branch 2 | 10 | Users | 10.30.10.0/24 | 10.30.10.1 |
| Branch 2 | 20 | Servers | 10.30.20.0/24 | 10.30.20.1 |
| HQ | - | SSL-VPN client pool | 10.99.99.10 - 10.99.99.50 | - |

## 4.3 End Devices

| Device | IP address | Gateway |
| --- | --- | --- |
| VPC17, VPC18 | DHCP (10.10.10.100 - 10.10.10.200) | 10.10.10.1 |
| VPC19 | 10.10.20.10/24 | 10.10.20.1 |
| VPC20 | 10.20.10.10/24 | 10.20.10.1 |
| VPC21 | 10.20.20.10/24 | 10.20.20.1 |
| VPC22 | 10.30.10.10/24 | 10.30.10.1 |
| VPC23 | 10.30.20.10/24 | 10.30.20.1 |
| Win | 5.5.5.2/24 | 5.5.5.1 |

## 4.4 Management Addresses (port1)

| Device | port1 address |
| --- | --- |
| Fortinet1 (HQ primary) | 192.168.77.151 |
| Fortinet2 (HQ secondary) | 192.168.77.152 |
| Fortinet4 (Branch 1) | 192.168.77.___ (fill from your lab) |
| Fortinet3 (Branch 2) | 192.168.77.___ (fill from your lab) |

On every FortiGate the default-gateway option of port1 is disabled so that the management DHCP lease never installs a default route that would bypass the WAN interface.

# 5. VLAN Design

| VLAN ID | Name | Sites | Tagged on |
| --- | --- | --- | --- |
| 10 | USERS | HQ, Branch 1, Branch 2 | FortiGate port4 (HQ) / port3 (branches) and all trunk links |
| 20 | SERVERS | HQ, Branch 1, Branch 2 | Same as above |
| 30 | MGMT | HQ only | Same as above |

Each VLAN has its own subnet, is tagged with 802.1Q on the trunks and terminates on a FortiGate VLAN sub-interface. The FortiGate is the only Layer 3 device for the VLANs.

# 6. ISP and Edge Router Configuration

| Router | Configuration summary |
| --- | --- |
| vIOS6 (ISP-HQ) | Gi0/0 203.0.1.1/30, Gi0/1 1.1.1.1/30. Static route 203.0.10.0/29 via 203.0.1.2. Default route via 1.1.1.2. |
| vIOS5 (Edge-HQ) | Gi0/0 203.0.1.2/30, Gi0/1 203.0.10.1/29. Default route via 203.0.1.1. |
| vIOS7 (ISP-B1 / Internet core) | Loopback0 8.8.8.8/32, Gi0/0 203.0.2.1/30, Gi0/1 1.1.1.2/30, Gi0/2 1.1.2.1/30. Static routes to 203.0.1.0/30, 203.0.10.0/29, 203.0.20.0/30, 203.0.3.0/30, 203.0.30.0/30 and 5.5.5.0/24. |
| vIOS8 (Edge-B1) | Gi0/0 203.0.2.2/30, Gi0/1 203.0.20.1/30. Default route via 203.0.2.1. |
| vIOS9 (ISP-B2) | Gi0/0 203.0.3.1/30, Gi0/1 1.1.2.2/30, Gi0/2 5.5.5.1/24. Static route 203.0.30.0/30 via 203.0.3.2. Default route via 1.1.2.1. |
| vIOS10 (Edge-B2) | Gi0/0 203.0.3.2/30, Gi0/1 203.0.30.1/30. Default route via 203.0.3.1. |

# 7. Switch Configuration

| Switch | Configuration |
| --- | --- |
| Switch11 (WAN) | Unmanaged, all ports in VLAN 1. It only bridges the edge router and both FortiGate WAN ports. |
| Switch12 (Core) | VLANs 10, 20, 30. Gi0/0 to Gi0/3 are 802.1Q trunks allowing VLANs 10, 20, 30. |
| Switch14 (Users) | Gi0/0 trunk (10, 20, 30). Gi0/1 and Gi0/2 access VLAN 10 with portfast. |
| Switch13 (Servers/Mgmt) | Gi0/0 trunk (10, 20, 30). Gi0/1 access VLAN 20 (VPC19). Gi0/2 access VLAN 30. |
| Switch15 (Branch 1) | VLANs 10, 20. Gi0/0 trunk (10, 20). Gi0/1 access VLAN 10. Gi0/2 access VLAN 20. |
| Switch16 (Branch 2) | Same layout as Switch15. |

Example configuration (Switch15):

vlan 10

name USERS

vlan 20

name SERVERS

interface Gi0/0

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk allowed vlan 10,20

interface Gi0/1

switchport mode access

switchport access vlan 10

spanning-tree portfast

interface Gi0/2

switchport mode access

switchport access vlan 20

spanning-tree portfast

# 8. Headquarters Configuration

## 8.1 High Availability (Active-Passive)

Two FortiGate units form a cluster. The primary processes all traffic while the secondary stays in standby. Configuration and sessions are synchronized over two heartbeat links, so if the primary fails or one of its monitored ports goes down, the secondary takes over automatically.

| Setting | Value |
| --- | --- |
| Mode | Active-Passive |
| Group name | HQ-HA |
| Heartbeat interfaces | port3 (priority 100), port5 (priority 50) |
| Monitored interfaces | port2 (WAN), port4 (LAN trunk) |
| Device priority | Fortinet1 = 200 (preferred primary), Fortinet2 = 100 |
| Session pickup | Enabled |
| Management interface reservation | port1 on both units, so each unit keeps its own management address |
| Override | Disabled; a recovered unit rejoins as secondary and does not cause a second interruption |

## 8.2 Failover Test

- A continuous ping to 8.8.8.8 was started from VPC17.

- Fortinet1 (the primary) was stopped.

- Fortinet2 detected the loss of Fortinet1 and was elected primary; the cluster log shows Fortinet1 as lost.

- The ping resumed after a short interruption, proving traffic continuity, and sessions were carried over because session pickup is enabled.

- Fortinet1 was started again and rejoined the cluster as secondary with the configuration in-sync.

## 8.3 Interfaces

| Interface | Role | Address | Notes |
| --- | --- | --- | --- |
| port1 | Management | 192.168.77.151 / .152 | HA reserved management, one address per unit |
| port2 | WAN | 203.0.10.2/29 | Alias WAN, administrative access: ping |
| port3, port5 | HA heartbeat | none | Direct links between the units |
| port4 | LAN trunk | none | Carries VLANs 10, 20, 30 |
| VLAN10-USERS | LAN | 10.10.10.1/24 | DHCP server enabled |
| VLAN20 (Servers) | LAN | 10.10.20.1/24 | The sub-interface is named VLAN20-USERS on the device but carries the Servers VLAN |
| VLAN30-MGMT | LAN | 10.10.30.1/24 | ping, https, ssh |

## 8.4 DHCP for the User VLAN

| Setting | Value |
| --- | --- |
| Interface | VLAN10-USERS |
| Address range | 10.10.10.100 - 10.10.10.200 |
| Default gateway | 10.10.10.1 |
| DNS server | 8.8.8.8 |

## 8.5 Inter-VLAN Routing

All inter-VLAN traffic passes through the HQ FortiGate, where policies decide what is allowed (see Section 12). Users can reach servers only on web and ping services, servers cannot initiate connections to users, and users cannot reach the management VLAN.

# 9. Branch Configuration

## 9.1 Branch 1 (Fortinet4)

| Item | Value |
| --- | --- |
| Hostname | FGT-Branch1 |
| port2 (WAN) | 203.0.20.2/30, gateway 203.0.20.1 |
| B1-USERS | VLAN 10 on port3, 10.20.10.1/24 |
| B1-SERVERS | VLAN 20 on port3, 10.20.20.1/24 |
| Default route | 0.0.0.0/0 via 203.0.20.1 on port2 |
| VPN routes | 10.10.0.0/16, 10.30.0.0/16 and 10.99.99.0/24 via B1-HQ |

## 9.2 Branch 2 (Fortinet3)

| Item | Value |
| --- | --- |
| Hostname | FGT-Branch2 |
| port2 (WAN) | 203.0.30.2/30, gateway 203.0.30.1 |
| B2-USERS | VLAN 10 on port3, 10.30.10.1/24 |
| B2-SERVERS | VLAN 20 on port3, 10.30.20.1/24 |
| Default route | 0.0.0.0/0 via 203.0.30.1 on port2 |
| VPN routes | 10.10.0.0/16, 10.20.0.0/16 and 10.99.99.0/24 via B2-HQ |

# 10. Routing

Routing is static and documented per site. Every FortiGate has a default route towards its edge router and routes for all remote private subnets through the matching IPsec tunnel interface.

| Device | Destination | Next hop / interface | Purpose |
| --- | --- | --- | --- |
| HQ FortiGate | 0.0.0.0/0 | 203.0.10.1 via port2 | Internet |
| HQ FortiGate | 10.20.0.0/16 | tunnel HQ-B1 | Branch 1 LANs |
| HQ FortiGate | 10.30.0.0/16 | tunnel HQ-B2 | Branch 2 LANs |
| Fortinet4 | 0.0.0.0/0 | 203.0.20.1 via port2 | Internet |
| Fortinet4 | 10.10.0.0/16, 10.30.0.0/16, 10.99.99.0/24 | tunnel B1-HQ | HQ, Branch 2, remote users |
| Fortinet3 | 0.0.0.0/0 | 203.0.30.1 via port2 | Internet |
| Fortinet3 | 10.10.0.0/16, 10.20.0.0/16, 10.99.99.0/24 | tunnel B2-HQ | HQ, Branch 1, remote users |
| vIOS5, vIOS8, vIOS10 | 0.0.0.0/0 | Towards the ISP router | Internet |
| vIOS6, vIOS7, vIOS9 | Site WAN subnets | Towards the matching edge router | Return traffic |

# 11. IPsec Site-to-Site VPN

## 11.1 Parameters

The same parameters are used on both ends of every tunnel; any mismatch would prevent the tunnel from establishing.

| Parameter | Value |
| --- | --- |
| IKE version | 1 (Main mode) |
| Authentication | Pre-shared key |
| Pre-shared key | 123456 |
| Phase 1 proposal | DES / MD5 |
| Diffie-Hellman group | 14 |
| NAT traversal | Disabled (no NAT between the sites) |
| Phase 2 proposal | DES / MD5 |
| Phase 2 selectors | 0.0.0.0/0 to 0.0.0.0/0 (route-based; static routes steer the traffic) |

Note: the project brief does not mandate an IKE version or algorithm, so these lab values were selected. In a production network AES256 with SHA256 and IKEv2 would be preferred, because DES and MD5 are considered weak.

## 11.2 Tunnels

| Tunnel | Device | Local WAN | Remote WAN | Purpose |
| --- | --- | --- | --- | --- |
| HQ-B1 | HQ FortiGate | 203.0.10.2 | 203.0.20.2 | HQ to Branch 1 |
| B1-HQ | Fortinet4 | 203.0.20.2 | 203.0.10.2 | Branch 1 to HQ |
| HQ-B2 | HQ FortiGate | 203.0.10.2 | 203.0.30.2 | HQ to Branch 2 |
| B2-HQ | Fortinet3 | 203.0.30.2 | 203.0.10.2 | Branch 2 to HQ |

## 11.3 Branch-to-Branch Communication

The two branches are not connected directly. Branch 1 traffic to Branch 2 enters the B1-HQ tunnel, is routed by the HQ FortiGate, and leaves through the HQ-B2 tunnel (and the reverse). This works because of three elements:

- Each branch has a static route to the other branch subnet through its tunnel to HQ.

- The HQ FortiGate has routes for both branch subnets through the corresponding tunnels.

- An HQ policy allows traffic between HQ-B1 and HQ-B2 in both directions, with security profiles applied.

# 12. Firewall Policies

Policies follow the least-privilege principle: each rule names the exact source and destination interfaces and only the services required. Traffic without a matching policy is dropped by the implicit deny.

## 12.1 HQ

| # | Name | From | To | Service | NAT | Action |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | USERS-to-WAN | VLAN10-USERS | port2 | HTTP, HTTPS, DNS, PING | Yes | Accept + profiles |
| 2 | SERVERS-to-WAN | VLAN20 | port2 | HTTP, HTTPS, DNS, PING | Yes | Accept + profiles |
| 3 | USERS-to-SERVERS | VLAN10-USERS | VLAN20 | HTTP, HTTPS, PING | No | Accept + IPS |
| 4 | MGMT-to-SERVERS | VLAN30-MGMT | VLAN20 | SSH, HTTPS, PING | No | Accept + IPS |
| 5 | DENY-SERVERS-to-USERS | VLAN20 | VLAN10-USERS | ALL | No | Deny, log |
| 6 | DENY-USERS-to-MGMT | VLAN10-USERS | VLAN30-MGMT | ALL | No | Deny, log |
| 7 | WAN-to-VIP-WEB | port2 | VLAN20 | TCP 8080 (VIP) | No | Accept + IPS, AV |
| 8 | LAN-to-VPN | HQ LAN VLANs | HQ-B1, HQ-B2 | ALL | No | Accept + profiles |
| 9 | VPN-to-LAN | HQ-B1, HQ-B2 | HQ LAN VLANs | ALL | No | Accept + profiles |
| 10 | B1-to-B2 via HQ | HQ-B1 and HQ-B2 | HQ-B2 and HQ-B1 | ALL | No | Accept + profiles |
| 11 | SSLVPN-to-internal | ssl.root | LAN VLANs, HQ-B1, HQ-B2 | ALL | No | Accept + profiles |

## 12.2 Branch 1

| # | Name | From | To | Service | NAT |
| --- | --- | --- | --- | --- | --- |
| 1 | B1-USERS-to-WAN | B1-USERS | port2 | HTTP, HTTPS, DNS, PING | Yes |
| 2 | B1-SERVERS-to-WAN | B1-SERVERS | port2 | HTTP, HTTPS, DNS, PING | Yes |
| 3 | LAN-to-B1-HQ | B1-USERS, B1-SERVERS | B1-HQ | ALL | No |
| 4 | B1-HQ-to-LAN | B1-HQ | B1-USERS, B1-SERVERS | ALL | No |

## 12.3 Branch 2

| # | Name | From | To | Service | NAT |
| --- | --- | --- | --- | --- | --- |
| 1 | B2-USERS-to-WAN | B2-USERS | port2 | HTTP, HTTPS, DNS, PING | Yes |
| 2 | B2-SERVERS-to-WAN | B2-SERVERS | port2 | HTTP, HTTPS, DNS, PING | Yes |
| 3 | LAN-to-B2-HQ | B2-USERS, B2-SERVERS | B2-HQ | ALL | No |
| 4 | B2-HQ-to-LAN | B2-HQ | B2-USERS, B2-SERVERS | ALL | No |

# 13. NAT Configuration

## 13.1 Outbound NAT

Every LAN to WAN policy uses NAT with the outgoing interface address, so private addresses are translated to the WAN address of the site (203.0.10.2, 203.0.20.2 or 203.0.30.2). VPN policies have NAT disabled so private addressing is preserved across the tunnels.

## 13.2 Virtual IP (Port Forwarding)

An internal web server in the HQ Servers VLAN is published to the outside world with a VIP.

| Item | Value |
| --- | --- |
| VIP name | VIP-WEB |
| External interface and IP | port2, 203.0.10.3 |
| Mapped (internal) IP | 10.10.20.10 |
| Port forwarding | External TCP 8080 to internal TCP 80 |
| Policy | WAN-to-VIP-WEB (port2 to Servers VLAN, destination VIP-WEB, service TCP 8080, with IPS and Antivirus) |

# 14. Security Profiles

Four security profiles are applied to every LAN to WAN policy and every VPN to LAN policy.

| Profile | Configuration | Behavior demonstrated |
| --- | --- | --- |
| Antivirus (AV-Custom) | Scan and block on HTTP, FTP, SMTP, POP3, IMAP | EICAR test file is blocked |
| Web Filter (WF-Custom) | Static URL filter blocking *facebook.com; category filtering for Proxy Avoidance | Blocked site shows the FortiGate block page |
| IPS (IPS-Custom) | Block signatures of high and critical severity | Attack signatures logged and blocked |
| Application Control (APP-Block) | Block P2P and Proxy categories, allow the rest | Blocked applications logged |

Profiles are applied with flow-based inspection and certificate inspection, on the policies listed in Section 12.

# 15. Remote User VPN

A remote user on the Win machine (behind vIOS9) connects to the HQ FortiGate with SSL-VPN and can reach the HQ and both branch networks, because the branches have routes and policies for the SSL-VPN pool.

| Item | Value |
| --- | --- |
| VPN type | SSL-VPN tunnel mode with FortiClient |
| Listening interface / port | port2 (203.0.10.2), TCP 10443 |
| Client address pool | 10.99.99.10 - 10.99.99.50 |
| Authentication | Local user remote1 in group SSLVPN-USERS |
| Split tunnel | Enabled for 10.0.0.0/8 |
| Policy | ssl.root to LAN VLANs, HQ-B1 and HQ-B2, with security profiles |
| Branch support | Static route and policy for 10.99.99.0/24 through the branch tunnels |

# 16. Test Results

| # | Test | Expected result | Screenshot |
| --- | --- | --- | --- |
| 1 | VPC17 receives a DHCP address | Address in 10.10.10.100 - .200 | 8.7 |
| 2 | VPC17 to gateway, WAN router and 8.8.8.8 | Success | 13.2 |
| 3 | VPC17 to VPC19 | Success (permitted) | 8.8 |
| 4 | VPC19 to VPC17 | Blocked (segmentation) | 8.8, 12.4 |
| 5 | HA failover | Traffic continues, roles switch | 8.3 - 8.6 |
| 6 | Branch 1 users to the Internet | Success with NAT | 9.3 |
| 7 | Branch 2 users to the Internet | Success with NAT | 9.6 |
| 8 | IPsec HQ-Branch 1 and HQ-Branch 2 | Tunnels Up | 11.1 - 11.5 |
| 9 | Branch-to-HQ communication | Success | 11.6 |
| 10 | Branch 1 to Branch 2 through HQ | Success | 11.6 |
| 11 | VIP external access | Web page reachable on 203.0.10.3:8080 | 13.4 |
| 12 | Security profile blocking | Blocked and logged | 14.6 |
| 13 | Remote user VPN | Connected, internal access works | 15.3, 15.4 |

# 17. Challenges and Solutions

| Challenge | Cause | Solution |
| --- | --- | --- |
| Management IP vanished after enabling HA | Management interface reservation moves port1 settings under the HA configuration | Re-applied the port1 address and set the reserved management gateway from the CLI |
| Router and switch configuration lost after a restart | The running configuration was never saved to startup | Re-applied the configuration and saved with write memory after every change |
| VPC17 received no DHCP address | Switch ports and trunks were still in VLAN 1 | Configured the trunks and access VLANs on Switch12 and Switch14 |
| Branch default route left through port1 | The management DHCP lease installed a default gateway | Disabled the default-gateway option on port1 and created the default route on port2 |
| Branch 1 users had no Internet | The policy had the wrong ingress interface (B1-SERVERS instead of B1-USERS) | Corrected the source interface of the policy |
| Branch VPCs could not reach their gateway | Switch ports and trunk were not configured on the branch switch | Re-applied the access and trunk configuration |

# 18. Conclusion

The project delivers a segmented, redundant and secure enterprise network. HQ is protected by an Active-Passive FortiGate cluster with a verified failover, every site performs local inter-VLAN routing under least-privilege policies, the three sites are connected by IPsec tunnels with Branch-to-Branch traffic routed through HQ, internal services are published through a VIP, security profiles inspect Internet and VPN traffic, and a remote user can securely reach internal resources.